# Garber Square

Traffic Study

Conducted by Patrolman Raymond Tarino

The Ridgewood Police Traffic Safety Bureau

Observations June 8th 2017 – June 20th 2017

The Ridgewood Police Traffic Safety Bureau was asked to conduct a traffic study in the area of Garber Square. We attempted to locate and identify any areas that were being affected by heavy traffic volume, causing significant traffic delays.

Garber Square is a heavily traveled 2 lane roadway that connects the eastside and westside of Ridgewood at the railroad underpass.

The 2-lane roadway is separated by a 2-foot-tall, 4-foot-wide cement/cobble stone center median.

On average, 18,000 vehicles traverse Garber Square on a daily basis.

As per NJ Transit, there are on average 210 bus movements through Garber Square on a daily basis.

Garber Square Eastbound

- Roadway extends from West Ridgewood Avenue to North Broad Street

- 730 feet of roadway

- The roadway takes a sharp right turn towards the underpass

- The vehicle lane of travel varies from 16 feet wide at the straight away, to 21.7 feet wide at the right curve, to 11.5 feet wide under the train overpass

- The bike lane is 6 feet wide and extends 580 feet along the right shoulder, marked by green retroreflective paint

- A combined Right Turn Only Lane and Bike Lane extends 200 feet and is 11.5 feet wide.

- At North Broad Street, Garber Square eastbound becomes Franklin Avenue

Garber Square Westbound

- Roadway extends from North Broad Street to West Ridgewood Avenue

- 880 feet of roadway

- The roadway takes a sharp left turn after the underpass

- The vehicle lane of travel varies from 17.6 wide at the straight away, to 18.8 feet at the left curve, to 11.5 feet at Library Place.

- Library Place is a ONEWAY street that feeds into Garber Square West.

- The Bike Lane is 6 feet wide and extends 600 feet along the right shoulder, marked by green retroreflective paint.

- A combined Right Turn Only lane and Bike Lane extends 180 feet to West Ridgewood Avenue

- At West Ridgewood Avenue, Garber Square westbound becomes, Wilsey Square, which ultimately turns into Godwin Avenue.

Peak Volumes and Traffic Delays:

Westbound Garber Square:

11:00am – 12:00pm, 615 vehicle averages

2:00pm – 3:00pm, 750 vehicle averages

3:00pm – 4:00pm, 850 vehicle averages

4:00pm – 5:00pm, 815 vehicle averages

5:00pm – 6:00pm, 850 vehicle averages

3:45pm – The furthest backup for traffic westbound reached the driveway of Stop and Shop. After shadowing this vehicle, it took 2 light cycles to get through Franklin and Oak Street, 2 light cycles to get through Franklin and N. Broad Street, and 1 light cycle to get across West Ridgewood Avenue. The trip took a total of 5.5 minutes. By 4:00pm, traffic had eased and delays were minor if at all.

5:15pm – The furthest backup westbound reached Cottage Place. I shadowed the last vehicle stopped with all lights red and no traffic moving. It took over 6 minutes to complete the trip from Cottage Place to West Ridgewood Avenue. By 5:30, traffic had eased and delays were minor if at all.

3:05pm – 3:25pm - North Broad Street backs up to East Ridgewood Avenue with some vehicles waiting 3 traffic light cycles to cross Franklin Avenue. The average North Broad Street traffic cycle allows 10 vehicles to make a left turn.

Eastbound Garber Square:

7:00am – 8:00am: 716 vehicles averages

8:00am – 9:00am: 785 vehicles averages

1200pm – 1:00pm: 731 vehicles averages

3:00pm – 4:00pm: 714 vehicles averages

5:00pm – 6:00pm: 686 vehicles averages

Garber Square Eastbound traffic averaged 2 traffic light cycles during peak volume hours.  At times during 8:00am – 9:00am, traffic would back up to the West Ridgewood Avenue and Wilsey Square intersection but would only cause a momentary delay. Traffic seemed to always be moving due to the traffic light cycles.

Wilsey Square: did not experience any significant back up during the peak hours. Vehicles would make it through the intersection within 1 full traffic light cycle.

West Ridgewood Avenue:

From 7:25 am – 8:00am, West Ridgewood Avenue was observed backing up 550ft, or just before Heights Road. I observed vehicles waiting 3 to 4 complete traffic light cycles to complete their movement through the intersection of W. Ridgewood Avenue and Garber Square.

There are 2 heavily occupied NJ transit trains during this time frame. Pedestrian traffic is at its peak, (middle schoolers and commuters).  Vehicles exiting and entering Station Plaza are also at their peak during this time frame. This is also during George Washington Middle School drop off.  A large number of vehicles were seen exiting Washington Place onto West Ridgewood Avenue.

None of these delays were caused by a backup on Garber Square eastbound. Most vehicles on West Ridgewood Avenue during this hour were attempting to turn left onto Garber Square eastbound.  Due to heavy pedestrian travel over Garber Square from West Ridgewood Avenue, as well as a heavy presence of vehicles exiting Station Plaza from commuter drop off, much of the green light cycle for left turning vehicles was spent yielding to the right of way of vehicles and stopping for pedestrians.

Traffic Light Cycles:

Franklin Avenue / North Broad Street / Garber Square Intersection

Franklin Avenue West/Garber Square East

| Cycle | Seconds |
| --- | --- |
| Lead Green (westbound) | 10 |
| Green all ways | 43 |
| Amber | 4 |
| Red (all ways) | 2 |

North Broad Street/Ken Smith Lot

| Cycle | Seconds |
| --- | --- |
| Green | 43 |
| Amber | 4 |
| Red (all ways) | 2 |

W. Ridgewood Ave / Wilsey Square / Garber Square Intersection

Wilsey Square/Garber Square West

| Cycle | Seconds |
| --- | --- |
| Green | 57 |
| Amber | 4 |
| Red (all ways) | 2 |

West Ridgewood Ave/Station Plaza

| Cycle | Seconds |
| --- | --- |
| Green | 39 |
| Amber | 4 |
| Red (all ways) | 2 |

Crash Analysis:

Motor Vehicle crash reports were compiled from January 2012 to October 2014, prior to when the Garber Square Improvement Project was completed.  They will be compared to motor vehicle crash reports compiled from November 2014 to May 2017.  Areas of analysis consisted of the motor vehicle crashes that occurred at the intersection of West Ridgewood Avenue and Garber/Wilsey Square, the intersection of Franklin Avenue and North Broad Street, Garber Square westbound, as well as Garber Square eastbound.

Slight motor vehicle crashes are defined as those that occur causing little to no damage and having no reported injuries.

Reportable motor vehicle crashes are defined as those that cause more than $500 in damage or cause injury.

Jan. 2012 to Oct. 2014 MVC’s investigated by the Ridgewood Police Department

| MVC’s | 2012 | 2013 | thru 10/2014 |
| --- | --- | --- | --- |
| Reportable | 5 | 6 | 4 |
| Slight | 9 | 4 | 2 |
| Total | 14 | 10 | 6 |
|  |  | Total | 30 |

Significant findings during time period:

-7 reported crashes involving a vehicle turning left from W. Ridgewood Ave. onto Garber Square eastbound, not yielding to the right of way to vehicles exiting station Plaza.

-2 reported same direction sideswipes in marked travel lanes of Garber Square

-All rear end crashes reported caused by driver inattention

-1 reported injury, no transportation to hospital

Nov. 2014 to May 2017 MVC’s investigated by the Ridgewood Police Department

| MVC’s | from 11/2014 | 2015 | 2016 | 2017 |
| --- | --- | --- | --- | --- |
| Reportable | 1 | 6 | 4 | 3 |
| Slight | 1 | 10 | 4 | 3 |
| Total | 2 | 16 | 8 | 6 |
|  |  |  | Total | 32 |

Significant finding during time period:

-2 reported crashes involving a vehicle turning left from W. Ridgewood Ave. onto Garber Square eastbound, not yielding to the right of way to vehicles exiting station plaza

-3 of the 4 same direction sideswipes were reportedly caused by vehicles improperly using bike lanes to pass.

-All rear end crashes reported were caused by driver inattention, vehicles striking stopped cars at red light.

-1 pedestrian struck, pedestrian left scene prior to Emergency Personnel arrival

-7 reported injuries resulting in 0 transportations to a hospital.

Jan. 2012 to Oct. 2014 MVC’s investigated by the Ridgewood Police Department

| Crash Type | # of Crashes |
| --- | --- |
| West Ridgewood/Wilsey Intersection |  |
| Left Turn MVA | 7 |
| Rear End MVA | 4 |
| Sideswipe Same Direction | 1 |
| Sideswipe Opposite Direction | 1 |
| Right Angle | 1 |
| Pedestrian | 0 |
| Total | 14 |
| North Broad/Franklin Intersection |  |
| Left Turn | 2 |
| Rear End | 3 |
| Sideswipe Same Direction | 1 |
| Sideswipe Opposite Direction | 0 |
| Right Angle | 0 |
| Pedestrian | 0 |
| Total | 6 |
| Garber Square Westbound |  |
| Rear End | 1 |
| Sideswipe Same Direction | 0 |
| Fixed Object | 1 |
| Pedestrian | 0 |
| Total | 2 |
| Garber Square Eastbound |  |
| Rear End | 3 |
| Sideswipe Same Direction | 2 |
| Fixed Object | 3 |
| Pedestrian | 0 |
| Total | 8 |

Nov. 2014 to May 2017 MVC’s investigated by the Ridgewood Police Department

| Type of Crash | # of Crashes |
| --- | --- |
| West Ridgewood/Wilsey Intersection |  |
| Left Turn MVA | 4 |
| Rear End MVA | 5 |
| Sideswipe Same Direction | 2 |
| Sideswipe Opposite | 0 |
| Right Angle | 0 |
| Pedestrian | 0 |
| Total | 11 |
| North Broad/Franklin Intersection |  |
| Left Turn | 0 |
| Rear End | 3 |
| Sideswipe Same Direction | 2 |
| Sideswipe Opposite Direction | 0 |
| Right Angle | 0 |
| Pedestrian | 1 |
| Total | 6 |
| Garber Square Westbound |  |
| Rear End | 4 |
| Sideswipe Same Direction | 3 |
| Fixed Object | 2 |
| Pedestrian | 0 |
| Total | 9 |
| Garber Square Eastbound |  |
| Rear End | 4 |
| Sideswipe Same Direction | 4 |
| Fixed Object | 1 |
| Pedestrian | 0 |
| Total | 6 |

Recommendations:

During peak hours, there are short bursts of heavy vehicular volume that cause traffic to back up the entire length of Garber Square eastbound and westbound.  When backups extend through either signalized intersection, W. Ridgewood/Wilsey or N. Broad/Franklin, traffic delays can peak as well.

Garber Square westbound

Garber Square westbound saw the most significant delays and volume during the 3:o0pm - 4:00pm hour and the 5:00pm - 6:00pm hour.

Extending the Combined Right Turn Only/Bike Lane from 180 feet to 360 feet, (in front of Pease Library, after curve) would allow a queuing of a total of 18 cars (previously 9). (16 foot average car length, plus 2 foot gaps between each stopped cars) 360 feet/20 feet = 18 cars)

Garber Square eastbound

Garber Square eastbound saw less significant delays, but an extension of the Combined Right Turn Only/Bike lane from 148 feet to 205 feet, would allow an additional 3 cars to queue.

Both directions of Garber Square allow Right on Red. More vehicles in the right lanes, turning right on red, will allow more traffic to flow, easing delays.

West Ridgewood Avenue

West Ridgewood Avenue sees heavy pedestrian and vehicular traffic during the morning commute and middle school drop off.  A lead green arrow on the West Ridgewood side of the traffic light combined with a pedestrian countdown, would give vehicles turning left the ability to make the turn unobstructed by pedestrians crossing Garber Square or vehicles exiting Station Plaza.

The West Ridgewood Right Turn lane is 45 feet long. Removing the first 2 of the 15 minute parking spaces along the south curb of West Ridgewood Avenue would extend the right turn lane by 40 feet, allowing more cars to queue that are attempting to turn right onto Wilsey Square.  It would also give vehicles going straight into Station Plaza the ability to go around stopped vehicles waiting to turn left onto Garber Square from West Ridgewood Avenue.

I would not change the light cycles at any locations as they appear to be maximizing their efficiency.